VMC ACCUVMC-300SE

| NO. | ITEM |  |  |  |  | Specification |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Table size L(mm)×W(mm) |  |  |  |  | 690×210 |
| 2 | T slot QTY /width/center distance(mm) |  |  |  |  | 3-16-63 |
| 3 | Maximum load(KG) |  |  |  |  | 100 |
| 4 | X travel (mm) |  |  |  |  | 330 |
| 5 | Y travel (mm) |  |  |  |  | 220 |
| 6 | Z travel (mm) |  |  |  |  | 320 |
| 7 | Spindle nose to table (mm) |  |  |  |  | 90-410 |
| 8 | Spindle center to column (mm) |  |  |  |  | 280 |
| 9 | Spindle taper |  |  |  |  | BT30 |
| 10 | Max. spindle speed (Rpm) |  |  |  |  | 10000 |
| 11 | Main motor （kW） |  |  |  |  | 2.2 |
| 12 | Feed motor | X torque（N.M） |  |  |  | 3.3 |
|  |  | Y torque（N.M） |  |  |  | 4 |
|  |  | Ztorque（N.M） |  |  |  | 5 |
|  |  | Max.install size |  |  | X | Square 90×90diameter φ80 |
|  |  |  |  |  | YZ | Square 110×110diameter φ95 |
| 13 | Rapid moving speed (m/min.） |  |  |  |  | 10 |
| 14 | Cutting speed （mm/min） |  |  |  |  | 1-4000 |
| 15 | Ball screw （diametermm/pitch ）mm |  |  |  |  | 20/5 |
| 16 | Linear guide way width（ mm） |  |  |  |  | 20 |
| 17 | Accuracy |  | Positioning accuracy (mm) |  |  | 0.02 |
|  |  |  | Repeatability accuracy (mm) |  |  | 0.01 |
| 18 | Tool changer |  | Funnel type (optional ) BT30 |  |  | 6 |
|  |  |  | Tool changing time (s) |  |  | 8 |
| 19 | Air pressure Mpa |  |  |  |  | 0.6 |
| 20 | Machine weight (kg) |  |  | Frame |  | 360 |
|  |  |  |  | Complete machine |  | 700 |
| 21 | Machine overall size : L(mm)×W(mm)×H(mm) |  |  | Frame |  | 720×860×1160 |
|  |  |  |  | Complete machine |  | 1200×1100×2000 |

Vertical cnc milling machine VMC ACCUMAX-SE

Able to mill steel, copper, aluminum, wood and plastic

Taiwan spindle, taiwan linear guide ways

Complete machine: standard with external handwheel ,tri-colour lights,lubrication,coolant system.

Complete machine standard with RS232 interface.

Frame is with lubrication, without controller, without any electrical parts.

Tool

Suggested max tool length(BT30):200mm

Suggested max tool weight(BT30):5kg

Suggested max tool diameter(BT30):16mm

ATC(automatic tool changer)

Standard:funnel type tool changer(BT30,8 tools)

Optional:manipulator tool changer(different price)

Controller

Stnadard:GSK218M or KND1000M2 controller (with rigid tappering fuction)

Optional:Siemens 808D control panel with servo motor and servo driver made in China

Motor

When machine is ATC,spindle motor is servo motor,2.2KW,torque 14NM

When machine is withour ATC,spindle motor is serve motor,2.2Kw,torque 15.5NM

X,Y,Z axes are all servo motors

Accessories

Complete machine is with standard accessories:1 pc tool holder,1 pc pull stud,1 set collects, 1 pc spanner, 1 pc direct screwdriver, 1 pc Philips screwdriver.

Power

Please inform power voltage and current,if transformer is needed price would be different.